SCHOOL OF ENGINEERING

APPENDIX A for RISK ASSESSMENT FOR AN ACTIVITY INVOLVING THE USE OF HAZARDOUS CHEMICALS

as required under the Control of Substances Hazardous to Health regulations

| (A) Health Hazard | (B) Dustiness or Volatility | (C) Quantity* | Score |
| --- | --- | --- | --- |
| Low Hazard statements (Risk phrases): H319 (R36); R36/38; H315 (R38). EUH066 (R66). H336 (R67). | Low Solids: Pellet-like solids that do not break up. Little or no dust observed during use. Solids forming large crystals. Liquids: Boiling point >150°C | Small <1g (mL) | 1 |
| Medium Hazard statements (Risk phrases): H332 (R20); H312 (R21); H302 (R22); H301 (R25); H314 (R34/35); H319 (R36); H335 (R37); H315 (R38); H318 (R41); H373 (R48); H304 (R65). | Medium Solids: Smaller crystalline or granular solids. Minimal dust, or if any dust is seen it settles out quickly. Liquids: Boiling point between 50 and 150°C. | Medium 1 to 100g (mL) | 2 |
| High Hazard statements (Risk phrases): H331 (R23); H311 (R24); H301 (R25); H330 (R26); H310 (R27); H300 (R28) H370 (R39); H334 (R42); H317 (R43); H350 (R45); H340 (R46); H372 (R48); H350i (R49); R42/43; R47; 48/23; 48/23/24; 48/23/25; H360F (R60); H360Fd (R61); H361f (R62); H360d (R63); H362 (R64); H341 (R68). | High Solids: Fine, light powders. Dust can be seen during use and possibly remains airborne for several minutes. Liquids: Boiling point <50°C. Gases. Aerosols. | Large >100g (mL) | 3 |

Multiply (A) × (B) × (C) to estimate overall risk level: ≤ 8 Low; 9-12 Medium; ≥ 13 High

*When stating quantity, this should consider the quantity in the stock bottle as well as the quantity of the aliquots, since loss of containment from the entire stock bottle whilst removing aliquots may represent the greatest risk.